HAMNET-RELAY DEV 2 · Packet Layer, Codec & TX Queue Phases 3, 4 & 5 · Builds 09 – 20 BUILD & TEST REQUIREMENTS SPECIFICATION · PRE-DEVELOPMENT DRAFT

## Document Overview

| Document | DEV 2 — Packet Layer, Codec & TX Queue |
| --- | --- |
| Phases Covered | Phase 3 (Builds 09–13) · Phase 4 (Builds 14–17) · Phase 5 (Builds 18–20) |
| Language | Rust (proposed) |
| Key Crates | serialport · cpal · ax25 · blake3 · rmp-serde · zstd · tokio · clap |
| Status | PRE-DEVELOPMENT DRAFT |
| Depends On | DEV 1 — Phase 1 & 2 phase gate must be fully passed |
| Output Of | Full two-station TX/RX loop · Identity system · Compressed payload codec · CSMA queue |

# 1. Purpose & Scope

This document defines the build requirements and test criteria for Phases 3, 4, and 5 of HAMNET-RELAY. These phases transform the working audio path from Dev 1 into a complete, protocol-correct radio data link: AX.25 packet framing, a binary identity and codec layer, and a CSMA-backed priority transmission queue.

By the end of Phase 5, the system is capable of transmitting compressed, identified, framed payloads over RF and receiving them at a remote station, with collision avoidance and priority scheduling. This is the foundation on which the HTTP API (Phase 6), image transfer (Phase 8), and RustChan integration (Phase 9) are built.

The mic/speaker input path from Phase 2 must remain fully operational throughout all builds in this document. No build may remove or break that capability.

# 2. Phase 3 — AX.25 Packet Layer

Phase 3 adds proper AX.25 framing around the raw AFSK audio path from Phase 2. AX.25 is the standard link-layer protocol for amateur packet radio and is required for FCC Part 97 compliance, since it carries the operator callsign as a mandatory field in every frame. Phase 3 also introduces optional PTT cable control as an enhancement to the mic/speaker mode.

| BUILD-09 | AX.25 Frame Assembler | Phase 3 | FEASIBLE |
| --- | --- | --- | --- |

Objective

Implement a standards-compliant AX.25 frame assembler. Every packet transmitted by HAMNET-RELAY must be wrapped in a valid AX.25 frame with source callsign, destination, optional digipeater path, payload, and FCS checksum. Output must be accepted by Direwolf and other standard AX.25 decoders.

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R09-01 | Must implement AX.25 frame structure: flag (0x7E), address fields (src/dst callsign + SSID), control byte (0x03 for UI frames), PID byte (0xF0 for no-layer-3), payload, FCS, flag. | MUST |
| R09-02 | Callsign fields must be encoded as 6 ASCII characters padded with spaces, each left-shifted by 1 bit per AX.25 spec. | MUST |
| R09-03 | SSID byte must be encoded as specified: bits 1–4 = SSID value, bit 5 = 1 if last address field, bit 6 = reserved 0, bit 7 = C-bit. | MUST |
| R09-04 | FCS must be computed as CRC-16-CCITT (polynomial 0x8408, reflected) over all address + control + PID + payload bytes. | MUST |
| R09-05 | Must implement bit-stuffing: insert a 0 bit after every run of five consecutive 1 bits in the frame body (not flags). | MUST |
| R09-06 | Source callsign must be configurable from config.toml (callsign + SSID fields). | MUST |
| R09-07 | Must support destination callsign APRS (for compatibility) and a custom HAMNET destination. | MUST |
| R09-08 | Assembled frame must be accepted by Direwolf 1.7+ without CRC error. | MUST |
| R09-09 | Assembler must return an Err if callsign is empty or longer than 6 characters. | MUST |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| Direwolf acceptance | Frame decoded by Direwolf with correct callsign and payload | direwolf -t 0 piped audio |
| CRC correctness | FCS bytes match reference implementation (AX.25 test vectors) | Unit test against known vectors |
| Bit stuffing | No run of more than 5 consecutive 1 bits in frame body | Unit test: inspect bit stream |
| Callsign too long | Returns Err("callsign exceeds 6 chars") | Unit test |
| Empty callsign | Returns Err("callsign must not be empty") | Unit test |
| APRS decode cross-check | Frame decoded by a second tool: multimon-ng or ax25-tools | ax25-tools decode on Linux |

| BUILD-10 | PTT Control via RTS/DTR Pin (Optional Enhancement) | Phase 3 | FEASIBLE |
| --- | --- | --- | --- |

Objective

Add support for hardware PTT control via the RTS or DTR pin of the USB serial cable. This is an enhancement to the mic/speaker path. If no cable is present, the application falls back to the Phase 2 mic/speaker mode automatically.

NOTE  This build is additive. The mic/speaker code path from Phase 2 must continue to function without any modification if no cable is present or if ptt_mode = 'none' in config.

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R10-01 | Must detect presence of a serial port flagged as is_radio = true by BUILD-01. | MUST |
| R10-02 | If port present and ptt_mode = 'rts' in config: assert RTS high before TX, release after. | MUST |
| R10-03 | If port present and ptt_mode = 'dtr' in config: assert DTR high before TX, release after. | MUST |
| R10-04 | PTT must be asserted at least 50 ms before audio playback begins (TX delay). | MUST |
| R10-05 | PTT must be released at least 100 ms after audio playback ends (tail delay). | MUST |
| R10-06 | If ptt_mode = 'none' (default) or no cable detected: use Phase 2 speaker-only path. | MUST |
| R10-07 | PTT assert/release must be logged with timestamps. | SHOULD |
| R10-08 | PTT timeout: if PTT asserted for more than 30 seconds (transmit watchdog), force release and log error. | MUST |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| RTS assert on TX | Radio PTT LED illuminates on RTS high; goes out on RTS low | Physical radio + multimeter on RTS pin |
| TX delay timing | Audio starts 50–60 ms after PTT assert | Oscilloscope: RTS pin + audio output |
| Tail delay timing | PTT releases 100–120 ms after audio ends | Oscilloscope |
| Fallback — no cable | Mic/speaker path used; no error; log says 'PTT: none' | Unplug cable, launch app, issue TX |
| Transmit watchdog | PTT force-released after 30s; error logged | Unit test: mock serial, stall TX |

| BUILD-11 | Packet TX Pipeline | Phase 3 | PARTIAL |
| --- | --- | --- | --- |

Objective

Wire together: payload input → AX.25 frame assembly (Build 09) → AFSK modulation (Build 03) → audio playback (Build 06) → PTT control if available (Build 10). This is the complete single-direction transmission path.

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R11-01 | Must accept a payload as &[u8] and produce a complete transmitted frame with no manual steps. | MUST |
| R11-02 | Pipeline sequence: wrap payload → AX.25 frame → AFSK modulate → PTT assert (if cable) → play audio → PTT release. | MUST |
| R11-03 | Source callsign must be read from config; must refuse to TX if callsign is empty. | MUST |
| R11-04 | Pipeline must be callable as an async function: async fn transmit(payload: &[u8]) -> Result<()>. | MUST |
| R11-05 | Must log: TX start, AX.25 frame size (bytes), audio duration (ms), TX end. | MUST |
| R11-06 | Must not allow a second transmit() call to start while a transmit() is in progress (mutex guard). | MUST |
| R11-07 | Must support a --transmit-text CLI flag for manual test transmission. | MUST |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| Remote decode — text | Transmit 'HELLO WORLD'; Direwolf on remote PC decodes it with correct callsign | Direwolf on second machine |
| Concurrent TX rejected | Second transmit() call returns Err while first is in progress | Unit test: spawn two concurrent tasks |
| Empty callsign blocked | transmit() returns Err if callsign not set in config | Unit test |
| Log output | Log contains: frame size, audio duration, start/end timestamps | Log inspection |
| CLI flag | --transmit-text 'TEST' sends frame; exit 0 | CLI test |

| BUILD-12 | Packet RX Pipeline | Phase 3 | FEASIBLE |
| --- | --- | --- | --- |

Objective

Wire the receive path: audio capture source (mic or audio jack) → AFSK demodulation (Build 04) → AX.25 frame disassembly → CRC validation → payload extraction and delivery.

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R12-01 | Must accept audio from either: system mic (Phase 2 path) or audio input jack via cpal. | MUST |
| R12-02 | Must perform AX.25 frame disassembly: detect flags, extract address fields, control, PID, payload, FCS. | MUST |
| R12-03 | Must validate FCS against recomputed CRC-16-CCITT. Discard frame and log error on mismatch. | MUST |
| R12-04 | Must remove bit-stuffing from the demodulated bit stream before frame parsing. | MUST |
| R12-05 | On valid frame: extract source callsign, destination, payload bytes. Deliver via tokio mpsc channel. | MUST |
| R12-06 | Must handle frames that span multiple audio buffer chunks (frame reassembly across chunks). | MUST |
| R12-07 | Must log: source callsign, RSSI estimate if available, frame length, CRC pass/fail, timestamp. | MUST |
| R12-08 | Must not crash on malformed or partial frames — return Err and continue. | MUST |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| Decode from known station | APRS packet from live radio decoded with correct source callsign | Live radio on 144.800 + mic |
| CRC fail discard | Corrupted frame (bit flipped) discarded; error logged | Unit test: flip bit in frame bytes |
| Partial frame handling | No panic on truncated frame; Err returned; next frame decoded OK | Unit test: truncated byte slice |
| Multi-chunk frame | Frame split across two audio buffer boundaries decoded correctly | Unit test: 512-sample split |
| Channel delivery | Received payload appears on mpsc receiver within 500 ms of RX | Integration test with loopback audio |

| BUILD-13 | Duplex Test — Full Two-Station TX/RX Loop | Phase 3 | PARTIAL |
| --- | --- | --- | --- |

Objective

Prove that two instances of HAMNET-RELAY — or one instance and any compatible AX.25 station — can successfully exchange packets in both directions. This is the Phase 3 gate test.

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R13-01 | Station A must be able to transmit a PING packet and receive a PONG response from Station B. | MUST |
| R13-02 | Both stations may use mic/speaker mode OR PTT cable mode. Either combination must work. | MUST |
| R13-03 | Round-trip latency (TX complete to RX decoded) must be under 5 seconds at 1200 baud for a 64-byte payload. | MUST |
| R13-04 | Both stations must log: transmitted callsign, received callsign, payload, timestamp. | MUST |
| R13-05 | Test must pass with one HAMNET-RELAY instance and one Direwolf/APRS station as the counterpart. | MUST |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| PING/PONG round trip | A sends PING seq=1; B decodes, logs, sends PONG seq=1; A decodes PONG | Two radios; manual or automated |
| Callsign in both directions | Correct callsigns logged on both sides for TX and RX | Log comparison |
| Round-trip latency | Under 5 seconds for 64-byte payload at 1200 baud | Timestamp difference in logs |
| Cross-tool compatibility | HAMNET-RELAY station A exchanges frames with Direwolf station B | Direwolf as counterpart |
| Mic/speaker mode cross-check | Test passes with both stations in mic/speaker mode (no cables) | Full cable-free test |

# 3. Phase 4 — Identity, Codec & Compression

Phase 4 adds the application-layer protocol above AX.25. Rather than transmitting raw text, HAMNET-RELAY will now transmit structured binary payloads: MessagePack-serialised structs, zstd-compressed, with a prepended identity hash and sequence number. This is the format used by all higher-level features including RustChan post submission.

| BUILD-14 | Client Identity System | Phase 4 | FEASIBLE |
| --- | --- | --- | --- |

Objective

Generate and persist a client identity. Each instance of HAMNET-RELAY generates a unique 256-bit random seed on first run, stores it in config.toml, and derives a Blake3 hash that serves as the pseudonymous station identifier included in every packet header.

WARN  HAM law prohibits encryption. The identity hash is a one-way pseudonymous identifier only — it is NOT used for access control or content obfuscation. The FCC callsign must still be transmitted separately in plain text in the AX.25 source address field.

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R14-01 | On first run: generate 32 random bytes (OsRng or equivalent) as identity_seed. | MUST |
| R14-02 | Store identity_seed as hex string in [identity] section of config.toml. | MUST |
| R14-03 | On subsequent runs: load identity_seed from config; do not regenerate. | MUST |
| R14-04 | Compute Blake3 hash of identity_seed. This is the full identity hash (32 bytes). | MUST |
| R14-05 | For packet headers, use the first 8 bytes of the hash as the short identity (16 hex chars). | MUST |
| R14-06 | Expose identity_short() -> [u8; 8] and identity_full() -> [u8; 32] as public functions. | MUST |
| R14-07 | Identity derivation must be documented in a public comments section of the source file — open derivation per FCC transparency. | MUST |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| First run generates seed | config.toml contains identity_seed after first launch | File inspection after first run |
| Seed persists | Same identity hash on second and subsequent runs | Run app twice; compare hash output |
| Short identity length | identity_short() returns exactly 8 bytes | Unit test |
| Hash determinism | blake3(seed) == blake3(same_seed) always | Unit test: 1000 iterations |
| Seed not regenerated | Deleting binary and re-running with same config: same hash | Manual test |

| BUILD-15 | MessagePack Serialisation | Phase 4 | FEASIBLE |
| --- | --- | --- | --- |

Objective

Define the PostPayload struct that carries all application-layer data in a HAMNET-RELAY packet. Implement serialisation and deserialisation using rmp-serde (MessagePack). This struct is the unit of transmission for text posts, and is later extended to carry image metadata.

PostPayload Struct Definition

| Field | Type | Description |
| --- | --- | --- |
| thread_id | u32 | Target thread ID on the RustChan instance |
| board_id | String | Board slug (e.g. 'g', 'b', 'hamnet') |
| text | String | Post body text, UTF-8, max 500 bytes before compression |
| image | Option<Vec<u8>> | JPEG-encoded image bytes, or None |
| seq_num | u32 | Monotonically increasing sequence number per sender |
| identity_short | [u8; 8] | First 8 bytes of Blake3 identity hash (Build 14) |
| callsign | String | FCC callsign in plain text — MANDATORY for legal TX |
| timestamp | u64 | Unix epoch seconds at time of composition |

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R15-01 | PostPayload must be serialisable with #[derive(Serialize, Deserialize)] via serde. | MUST |
| R15-02 | MessagePack serialisation must use rmp-serde::to_vec_named() for forward compatibility. | MUST |
| R15-03 | Deserialisation must tolerate missing optional fields (serde default handling). | MUST |
| R15-04 | Serialised size of a minimal text-only post (500-char text) must be under 600 bytes before compression. | MUST |
| R15-05 | Must return Err if callsign field is empty — callsign is mandatory. | MUST |
| R15-06 | image field must be None by default. If present, it must be a valid JPEG header (0xFF 0xD8). | MUST |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| Round-trip — text post | serialize then deserialize: all fields match original | Unit test |
| Round-trip — with image | 1KB JPEG bytes survive serialize/deserialize unchanged | Unit test: compare byte slices |
| Size budget — text only | Serialised size of 500-char post < 600 bytes | Unit test: measure Vec<u8> length |
| Empty callsign rejected | serialize() returns Err if callsign == "" | Unit test |
| Missing optional field | Deserialise JSON missing 'image' field: image = None, no error | Unit test: partial MessagePack |

| BUILD-16 | zstd Compression | Phase 4 | FEASIBLE |
| --- | --- | --- | --- |

Objective

Apply zstd compression to the serialised MessagePack payload to reduce over-air transmission time. Measure and verify compression ratios for typical post payloads. This is critical because at 1200 baud every saved byte saves approximately 7 milliseconds of airtime.

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R16-01 | Must use the zstd Rust crate. Compression level must be configurable; default level 3. | MUST |
| R16-02 | Must compress a Vec<u8> payload and return Vec<u8> compressed bytes. | MUST |
| R16-03 | Must decompress compressed bytes and return the original Vec<u8>. | MUST |
| R16-04 | A 1000-byte repetitive text payload must compress to under 200 bytes at level 3. | MUST |
| R16-05 | A 500-byte random byte payload must still decompress correctly even if compressed size > original. | MUST |
| R16-06 | compress() and decompress() must be fallible (return Result) and not panic on malformed input. | MUST |
| R16-07 | Log original size, compressed size, ratio, and compression time (ms) for every compress() call in debug mode. | SHOULD |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| Round-trip text | decompress(compress(text_bytes)) == text_bytes | Unit test |
| Compression ratio | 1000-byte repetitive payload: compressed < 200 bytes | Unit test: measure length |
| Random data round-trip | compress then decompress random 500 bytes: identical output | Unit test with OsRng bytes |
| Corrupt input | decompress(garbage_bytes) returns Err, no panic | Unit test |
| Level config | Level 1 compresses faster than level 9; both round-trip OK | Benchmark: compare ns per call |

| BUILD-17 | Full Payload Codec Pipeline | Phase 4 | FEASIBLE |
| --- | --- | --- | --- |

Objective

Assemble the complete application-layer codec: PostPayload struct → MessagePack → zstd → prepend packet header (identity + seq_num + checksum) → AX.25 wrap → AFSK transmit. And the reverse: receive AX.25 → strip → decompress → deserialise → deliver PostPayload.

Packet Header Layout (Prepended to Compressed Payload)

| Bytes | Field | Description |
| --- | --- | --- |
| 0–7 | identity[8] | Short identity hash (Blake3 first 8 bytes, Build 14) |
| 8–11 | seq_num[4] | u32 big-endian sequence number |
| 12–15 | checksum[4] | CRC-32 of compressed payload bytes |
| 16+ | payload[N] | zstd-compressed MessagePack PostPayload |

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R17-01 | encode(post: &PostPayload) -> Result<Vec<u8>> must produce the exact byte layout above. | MUST |
| R17-02 | decode(bytes: &[u8]) -> Result<PostPayload> must validate checksum before decompressing. | MUST |
| R17-03 | If checksum fails in decode(), return Err and do not attempt decompression. | MUST |
| R17-04 | encode() then decode() on the same PostPayload must reproduce all fields identically. | MUST |
| R17-05 | encode() output must fit within a single AX.25 UI frame (max 256 bytes payload) for text-only posts. | MUST |
| R17-06 | For payloads over 256 bytes (e.g. with image), must split into multiple frames with chunk_index and total_chunks appended to header. | SHOULD |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| Full round-trip — text | encode then decode a text post: all PostPayload fields identical | Unit test |
| Checksum failure | Flip one byte in encoded output: decode returns Err | Unit test: corrupt byte 20 |
| Fits in AX.25 frame | encode(text_post) output <= 256 bytes | Unit test: measure length |
| Remote station decode | Transmit encoded post; station B reconstructs correct PostPayload | Two-radio end-to-end test |
| Callsign in AX.25 header | Decoded frame at station B shows correct source callsign | Direwolf decode inspection |

# 4. Phase 5 — CSMA & TX Queue

Phase 5 adds the channel access and transmission scheduling layer. Before this phase, the application transmits immediately on request. After this phase, every transmission goes through a CSMA carrier-sense check and a priority queue. This is essential for operating as a well-behaved station on a shared amateur frequency.

| BUILD-18 | Channel Carrier Sense | Phase 5 | FEASIBLE |
| --- | --- | --- | --- |

Objective

Provide a persistent real-time channel state (BUSY / CLEAR) derived from audio input level, usable by the TX pipeline to implement listen-before-transmit. This extends Build 07 (VOX level detection) with shareable atomic state and integration into the TX path.

NOTE  This build replaces the standalone VOX detection from Build 07. Build 07 becomes a unit test fixture. The carrier sense in this build is the authoritative channel state for all TX decisions.

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R18-01 | Must maintain a shared AtomicBool (channel_busy) visible to all async tasks. | MUST |
| R18-02 | Audio monitor thread must update channel_busy based on 50 ms RMS window (per Build 07 algorithm). | MUST |
| R18-03 | Squelch threshold and hysteresis timing must be configurable in config.toml. | MUST |
| R18-04 | channel_busy must become true within 100 ms of a signal exceeding squelch. | MUST |
| R18-05 | channel_busy must become false within 300 ms of signal dropping below squelch (hysteresis). | MUST |
| R18-06 | channel_busy must be false by default on startup (no false-trigger during init). | MUST |
| R18-07 | Must expose a pub fn is_channel_busy() -> bool callable from any module. | MUST |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| Busy → true on signal | channel_busy = true within 100 ms of tone above threshold | Play tone; poll atomic; measure time |
| Clear after signal ends | channel_busy = false within 300 ms of tone stopping | Stop tone; poll atomic; measure time |
| No false trigger on init | channel_busy = false for first 2 seconds of startup in quiet room | Automated: check flag at t=0, t=1, t=2 |
| Concurrent read safety | 100 concurrent reads of is_channel_busy() produce no data race | Unit test: 100 threads, Loom or MIRI |
| Config threshold respected | Higher squelch value in config reduces sensitivity | Config edit + replay of marginal-level audio |

| BUILD-19 | CSMA Listen-Before-Transmit | Phase 5 | FEASIBLE |
| --- | --- | --- | --- |

Objective

Implement the CSMA (Carrier Sense Multiple Access) protocol gate in the TX pipeline. Before any frame is transmitted, the pipeline must check channel_busy and wait with random backoff until the channel is clear.

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R19-01 | Before any TX: wait until channel_busy = false for a minimum dwell time of 500 ms. | MUST |
| R19-02 | If channel becomes busy during the dwell period, reset the dwell timer and wait again. | MUST |
| R19-03 | After a failed listen (channel was busy), apply random backoff: uniform random between 50 ms and 500 ms before re-checking. | MUST |
| R19-04 | Maximum backoff attempts before giving up and returning Err(ChannelBusy): 20 attempts (configurable). | MUST |
| R19-05 | Log each backoff with: attempt number, wait duration, channel state at re-check. | SHOULD |
| R19-06 | CSMA check must be atomic with TX start — no other coroutine may transmit between clear check and TX assert. | MUST |
| R19-07 | CSMA logic must work identically in both PTT cable mode and mic/speaker mode. | MUST |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| TX deferred on busy channel | TX request while channel busy: TX deferred until channel clear | Play continuous tone; send TX; confirm deferral |
| TX fires after clear | After tone stops, TX fires within 500 ms + backoff | Measure time from tone stop to TX start |
| No collision in 50-send test | 50 sequential TXs; 0 collisions detected | Two simulated stations; log analysis |
| Max backoff exhaustion | Returns Err after 20 failed attempts | Unit test: mock channel_busy always true |
| Log backoff events | Each backoff attempt logged with timestamp and duration | Log inspection after busy-channel test |

| BUILD-20 | Priority TX Queue | Phase 5 | FEASIBLE |
| --- | --- | --- | --- |

Objective

Replace direct transmit() calls with a priority-ordered queue. Callers push items into the queue and a dedicated TX worker pops the highest-priority item after a CSMA check. This decouples the application from the radio and enables fair scheduling of different frame types.

Priority Levels

| Level | Name | Use Cases |
| --- | --- | --- |
| 0 (highest) | CRITICAL | Beacon / callsign ID (FCC required) · ACK frames |
| 1 | HIGH | SYNC handshake packets · PONG replies |
| 2 | MEDIUM | Text post payloads (PostPayload with no image) |
| 3 (lowest) | LOW | Image chunks · File transfer chunks |

Requirements

| Req ID | Requirement | Priority |
| --- | --- | --- |
| R20-01 | Must implement a priority queue with 4 levels (0 = highest). Use a binary heap or channel-per-priority approach. | MUST |
| R20-02 | TX worker must always pop the highest-priority non-empty level before lower levels. | MUST |
| R20-03 | Queue must be backed by async channels (tokio mpsc or similar) — no blocking. | MUST |
| R20-04 | Queue depth (total items across all levels) must be readable via a queue_depth() -> usize function. | MUST |
| R20-05 | Each queued item must have a unique ID (u64) and a timestamp_queued field for latency tracking. | MUST |
| R20-06 | Queued items must be cancellable by ID before they are popped by the TX worker. | MUST |
| R20-07 | TX worker must run CSMA check (Build 19) before each dequeued transmission. | MUST |
| R20-08 | Must log: item enqueued (ID, priority, estimated size), item dequeued, TX complete with latency. | MUST |
| R20-09 | Queue capacity must be configurable (default: 64 items total). Return Err(QueueFull) on overflow. | SHOULD |

Test Criteria

| Test Item | Pass Criteria | Tool / Method |
| --- | --- | --- |
| Priority ordering | Flood queue with 10 LOW items, insert 1 HIGH: HIGH transmits first | Unit test: track transmission order |
| CRITICAL before all | CRITICAL item inserted last transmits before queued MEDIUM items | Unit test |
| Queue depth readable | queue_depth() returns correct count after 5 enqueues and 2 dequeues | Unit test |
| Cancel before TX | Cancel queued item by ID: item never transmitted | Unit test: enqueue, cancel, drain queue |
| Queue full returns Err | Enqueuing 65th item (default cap 64) returns Err(QueueFull) | Unit test |
| CSMA on every TX | Introduce busy channel: all queued items wait, then transmit in priority order when clear | Integration test |
| Latency log | Log contains enqueue timestamp, dequeue timestamp, TX complete; latency computable | Log inspection |

# 5. Rust Crate Dependencies

| Req ID | Requirement | Priority |
| --- | --- | --- |
| serialport | Serial PTT control (RTS/DTR). Already in Phase 1. | MUST |
| cpal | Audio I/O. Already in Phase 1. | MUST |
| blake3 | Identity hash derivation (Build 14). | MUST |
| rmp-serde | MessagePack serialisation of PostPayload (Build 15). | MUST |
| serde | Derive macros for all serialisable structs. | MUST |
| zstd | Compression of serialised payload (Build 16). | MUST |
| crc32fast | CRC-32 checksum in packet header (Build 17). | MUST |
| tokio | Async runtime, mpsc channels for TX queue and RX delivery. | MUST |
| rand | Random backoff duration generation in CSMA (Build 19). OsRng for identity seed. | MUST |
| clap | CLI flags. Already in Phase 2. | MUST |
| proptest | Property-based testing for codec round-trips. | SHOULD |
| loom | Concurrency testing for atomic channel_busy flag (Build 18). | OPTIONAL |

# 6. Phase Gate — Criteria to Advance to Phase 6 (HTTP/WS API)

All items below must be confirmed before beginning Phase 6. Phase 6 exposes the TX queue, RX buffer, and radio state via a local HTTP API — all three of which must be stable and tested before wrapping in an API layer.

- BUILD-09: AX.25 frames accepted by Direwolf with correct callsign and FCS.

- BUILD-10: PTT cable path functional; mic/speaker fallback confirmed still working.

- BUILD-11: End-to-end TX pipeline transmits text; remote station decodes with correct callsign.

- BUILD-12: RX pipeline decodes live APRS packets from a real radio via microphone.

- BUILD-13: Two-station PING/PONG round trip: sub-5-second latency; callsigns correct both ways.

- BUILD-14: Identity hash stable across restarts; correctly prepended to all TX packets.

- BUILD-15: PostPayload serialises and deserialises with all fields intact; callsign validation works.

- BUILD-16: zstd round-trips correctly; 1000-byte repetitive payload compresses below 200 bytes.

- BUILD-17: Full codec pipeline encodes and decodes a PostPayload end-to-end over two radios.

- BUILD-18: channel_busy atomic flag responds within 100 ms; no false triggers in quiet room.

- BUILD-19: CSMA defers TX correctly; 50-send collision-free test passes.

- BUILD-20: Priority queue transmits CRITICAL before HIGH before MEDIUM before LOW. Cancel and depth functions work.

NOTE  The mic/speaker path must still pass Build 08 integration test criteria after all Phase 3–5 work. Regression test this before marking the phase gate complete.

# 7. FCC Part 97 Compliance Reminders for Phases 3–5

WARN  All AX.25 frames transmitted in any test must include a valid FCC callsign in the source address field. Even bench tests over RF require identification. Use a NOCALL placeholder only for audio loopback tests where no RF is emitted.

- Callsign must be transmitted at least every 10 minutes during operation — the beacon built in Phase 7 covers this, but during Phase 3–5 testing you must manually ensure identification.

- No encryption of payload content. zstd compression and MessagePack encoding are encoding formats, not encryption. The open-source nature of the codec is the compliance basis.

- No commercial use. All bench testing must remain non-commercial.

- AFSK 1200 baud Bell 202 = F2D emission type. Verify this is permitted on your chosen test frequency and license class.

HAMNET-RELAY  ·  DEV 2  ·  PHASES 3, 4 & 5  ·  PRE-DEVELOPMENT DRAFT  ·  73 DE HAMNET